Pi Radio Station Player Installation & Setup Guide v41 — Including Streaming Setup

A complete guide to installing, configuring, and running

Pi Radio Station Player on Raspberry Pi OS or any Debian/Ubuntu Linux desktop.

# Overview

Pi Radio Station Player is a full-featured radio automation system that runs on a Raspberry Pi or any Debian/Ubuntu Linux desktop. It plays audio files from a local music library, supports scheduled programming blocks, hourly chimes, commercial breaks, and fade transitions. It also includes a built-in streaming companion (pi_stream) that captures your audio output and broadcasts it live over the internet via Icecast2, so listeners anywhere can tune in.

## What Gets Installed

The installer sets up everything in one go — no separate steps required:

| Component | Purpose |
| --- | --- |
| Python 3 + pip | Core runtime and package installer |
| tkinter | Desktop UI library for the radio player and stream controller |
| VLC + python-vlc | Audio playback engine |
| PulseAudio + pactl | Audio routing — required by pi_stream to detect audio sources |
| darkice | Captures system audio and encodes it for Icecast streaming |
| icecast2 | The streaming server — listeners connect here on port 8000 |
| Program files | All .py, .sh, .html, and .cfg files copied to ~/radio |

## System Requirements

| Component | Requirement |
| --- | --- |
| Operating System | Raspberry Pi OS (Bullseye or Bookworm) or Debian/Ubuntu desktop |
| Python | 3.9 or newer |
| VLC | Any recent version (3.x or 4.x) |
| Display | Required for the desktop UI (desktop environment or X11 forwarding) |
| Audio Output | Speakers or headphones connected to the Pi |
| Storage | USB drive, SD card, or hard drive containing your audio library |
| Network | Required if using streaming — listeners need to reach port 8000 |

## File Overview

| File | Purpose |
| --- | --- |
| run_radio.py | Recommended launcher — starts backend then opens the UI |
| radio_backend.py | The backend engine (audio engine + HTTP API) |
| radio_ui.py | The desktop GUI |
| shared_radio.py | Shared constants and settings dataclass |
| start_all.sh | Shell-script alternative to run_radio.py |
| start_backend.sh | Start backend only, without launching the UI |
| kill_radio.sh | Stop all radio processes cleanly |
| pi_radio_web.html | Browser-based web UI (open in any browser) |
| pi_stream.py | Streaming companion — captures audio and sends to Icecast |
| pi_stream_darkice_v2.cfg | DarkIce streaming configuration (auto-managed by pi_stream) |
| backend.out | Backend log file — check here if something goes wrong |

# Quick Install (Recommended)

The installer handles everything — core packages, streaming software, Icecast2 configuration, and all program files — in a single run.

## How to Run It

Place install_pi_radio.sh in the same folder as all the radio program files, then run:

bash install_pi_radio.sh

The installer will work through 10 steps automatically:

- Check sudo access (prompts once at the start)

- Update package lists

- Install Python 3, pip, tkinter, VLC, libvlc-dev, PulseAudio, and utilities

- Install darkice and icecast2, enable and start the icecast2 service

- Install python-vlc via pip

- Configure Icecast2 — sets passwords and opens to all network interfaces

- Verify every dependency with a pass/fail report

- Copy all 10 program files to ~/radio

- Check audio output configuration

- Optionally install a systemd boot service

## Optional Flags

| Flag | What it does |
| --- | --- |
| --autostart | Install a systemd service so the backend starts automatically on boot |
| --dir=/path/to/folder | Install program files to a custom folder (default: ~/radio) |

Example — custom folder with autostart:

bash install_pi_radio.sh --dir=/home/pi/myradio --autostart

The installer pre-answers the icecast2 setup wizard silently — you will not be interrupted by any configuration prompts.

# Manual Installation (Step by Step)

Follow these steps in order if you prefer to install each component yourself, or need to troubleshoot a specific part of the setup.

| 1 | Update your system |
| --- | --- |

sudo apt update && sudo apt upgrade -y

| 2 | Install Python 3 and pip |
| --- | --- |

sudo apt install -y python3 python3-pip

| 3 | Install tkinter (the desktop UI library) |
| --- | --- |

Required by both the radio player UI and the pi_stream streaming controller:

sudo apt install -y python3-tk

| 4 | Install VLC and the VLC development headers |
| --- | --- |

sudo apt install -y vlc libvlc-dev

| 5 | Install PulseAudio |
| --- | --- |

PulseAudio is required by pi_stream.py to detect audio sources and route audio to darkice. pactl is the command-line tool it uses to list available sources:

sudo apt install -y pulseaudio pulseaudio-utils

PulseAudio starts automatically when you log into the desktop. If you run pi_stream.py over SSH without a desktop session, PulseAudio may not be running.

| 6 | Install darkice and icecast2 |
| --- | --- |

darkice captures your system audio and encodes it into an MP3 stream. icecast2 is the server that delivers the stream to listeners:

sudo apt install -y darkice icecast2

# Enable and start the icecast2 service

sudo systemctl enable icecast2

sudo systemctl start  icecast2

⚠  The icecast2 installer may show a setup wizard. It is safe to accept all defaults — the pi_stream UI lets you change passwords later.

| 7 | Configure Icecast2 |
| --- | --- |

Set the Icecast2 passwords to match the pi_stream defaults so streaming works immediately without any manual editing:

sudo nano /etc/icecast2/icecast.xml

Find these lines and set the passwords to hackme (or any password you prefer — just make sure pi_stream uses the same one):

<source-password>hackme</source-password>

<relay-password>hackme</relay-password>

<admin-password>hackme</admin-password>

# Also change bind-address to allow LAN/Tailscale listeners:

<bind-address>0.0.0.0</bind-address>

Then restart Icecast2 to apply the changes:

sudo systemctl restart icecast2

⚠  Change the default password 'hackme' to something stronger if this Pi is accessible from the internet.

| 8 | Install python-vlc |
| --- | --- |

pip3 install python-vlc --break-system-packages

⚠  On Raspberry Pi OS Bookworm, if you get an 'externally managed environment' error, add --user:

pip3 install python-vlc --break-system-packages --user

| 9 | Verify everything installed correctly |
| --- | --- |

Run this script to confirm all components are working:

python3 - << 'EOF'

import sys

print(f'Python: {sys.version}')

try: import tkinter;  print('tkinter:    OK')

except Exception as e: print(f'tkinter:    MISSING — {e}')

try: import vlc;      print('python-vlc: OK')

except Exception as e: print(f'python-vlc: MISSING — {e}')

try:

import vlc; inst = vlc.Instance()

print('VLC engine: OK')

except Exception as e: print(f'VLC engine: MISSING — {e}')

EOF

# Check streaming tools

command -v darkice    && echo 'darkice:  OK' || echo 'darkice:  MISSING'

command -v icecast2   && echo 'icecast2: OK' || echo 'icecast2: MISSING'

command -v pactl      && echo 'pactl:    OK' || echo 'pactl:    MISSING'

systemctl is-active icecast2 && echo 'icecast2 service: running' || echo 'icecast2 service: stopped'

| 10 | Copy program files and run |
| --- | --- |

mkdir -p ~/radio

cp *.py *.sh *.html *.cfg ~/radio/

chmod +x ~/radio/start_all.sh ~/radio/start_backend.sh ~/radio/kill_radio.sh

# Start the radio player

cd ~/radio && python3 run_radio.py

# In a second terminal — start the streaming controller

cd ~/radio && python3 pi_stream.py

✔  run_radio.py and pi_stream.py are independent programs. You can run one without the other.

# Streaming Setup Guide

Pi Radio includes a streaming companion called pi_stream. It captures whatever audio your Pi is playing and broadcasts it live to Icecast2, so anyone on your network (or over Tailscale) can listen in a browser or media player.

📡  pi_stream.py is completely independent of the radio player. It works by monitoring your Pi's audio output — it does not matter what is producing the sound.

## How It Works

| Component | Role |
| --- | --- |
| VLC / Radio | Plays audio to your Pi's sound output |
| PulseAudio | Routes audio between applications — pi_stream taps in here |
| darkice | Captures the PulseAudio monitor source and encodes to MP3 |
| icecast2 | Receives the darkice stream and serves it to listeners on port 8000 |
| pi_stream.py | The GUI that controls darkice — configure and start/stop streaming |

## Starting a Stream

- Make sure the radio player (or any audio) is playing on the Pi.

- Open a terminal and run:

cd ~/radio && python3 pi_stream.py

- In the pi_stream window, confirm the Icecast settings (host, port, password, mount point).

- Select the audio source — choose Auto-detect or pick the monitor source that shows ▶ ACTIVE.

- Click Start Streaming.

- The status dot will turn green and show LIVE when the stream is active.

## Listener Connection

Listeners connect to your Pi's IP address on port 8000. The default stream URL is:

http://<your-Pi-IP>:8000/stream

This URL works in any browser, VLC, or media player. To find your Pi's IP address:

hostname -I

📡  If you use Tailscale, listeners can connect from anywhere using your Pi's Tailscale IP address. No port forwarding required.

## Default Icecast2 Settings

| Setting | Default Value |
| --- | --- |
| Server | 127.0.0.1 (localhost — darkice connects here) |
| Port | 8000 |
| Mount point | stream |
| Password | hackme |
| Stream URL | http://<Pi-IP>:8000/stream |
| Admin page | http://<Pi-IP>:8000/admin |

⚠  Change the default password in /etc/icecast2/icecast.xml if this Pi is accessible from the internet. Update the password in the pi_stream UI to match.

## Audio Source Selection

When you open pi_stream, the source dropdown shows all available PulseAudio sources grouped by status:

- ▶ ACTIVE — sources currently receiving audio (this is what you want)

- 🔊 Output Monitor — loopback taps of audio output devices

- 🎤 Input — microphones, line-in, and USB audio inputs

Auto-detect (recommended) picks the best active monitor source automatically. If you have a specific Bluetooth speaker or HDMI output you always want to stream from, select it manually and pi_stream will remember it.

The pi_stream_darkice_v2.cfg file in the program folder contains a hardcoded Bluetooth device address. This is only used as a reference — pi_stream.py always generates a fresh config from the UI settings when you click Start Streaming.

# Autostart on Boot (Optional)

To have the radio backend start automatically every time the Pi boots, install a systemd service. The easiest way is to run the installer with the --autostart flag:

bash install_pi_radio.sh --autostart

Or install the service manually:

sudo tee /etc/systemd/system/pi-radio.service > /dev/null << 'SERVICE'

[Unit]

Description=Pi Radio Station Player Backend

After=network.target sound.target

[Service]

Type=simple

User=pi

WorkingDirectory=/home/pi/radio

ExecStart=/usr/bin/python3 -u radio_backend.py

Restart=on-failure

RestartSec=5

Environment=PYTHONUNBUFFERED=1

[Install]

WantedBy=multi-user.target

SERVICE

sudo systemctl daemon-reload

sudo systemctl enable pi-radio

sudo systemctl start  pi-radio

⚠  Change User=pi and WorkingDirectory= to match your actual username and install path.

## Service Management

| Command | What it does |
| --- | --- |
| sudo systemctl status pi-radio | Check whether the service is running |
| sudo systemctl stop pi-radio | Stop the backend |
| sudo systemctl start pi-radio | Start the backend |
| sudo systemctl disable pi-radio | Remove from boot (stops auto-starting) |
| tail -f ~/radio/backend.out | Watch the backend log in real time |

Icecast2 is set up as a service by the installer and starts on boot automatically — no extra configuration needed for streaming autostart.

# Audio Output Configuration

## Raspberry Pi — raspi-config

sudo raspi-config

Navigate to: System Options → Audio → select your output (HDMI or 3.5mm headphone jack).

## Testing Audio

aplay /usr/share/sounds/alsa/Front_Center.wav

Fix audio output before running the radio. VLC and darkice both use the same PulseAudio subsystem.

# Troubleshooting

The first place to look when something goes wrong is the backend log:

cat ~/radio/backend.out

For streaming problems, check the darkice log:

cat ~/.pi_stream_darkice.log

And the Icecast2 service log:

sudo journalctl -u icecast2 -n 50

## Radio Player Issues

### tkinter is not available

sudo apt install -y python3-tk

### python-vlc / VLC not available

sudo apt install -y vlc libvlc-dev

pip3 install python-vlc --break-system-packages

### No audio output

- Run sudo raspi-config → System Options → Audio → select output device

- Test with: aplay /usr/share/sounds/alsa/Front_Center.wav

### Backend failed to start

- Check cat ~/radio/backend.out — the last few lines show the error

- Most often caused by a missing python-vlc install

### UI window doesn't appear

- Must be run in a desktop environment, not a bare SSH session

- To forward the window over SSH:

ssh -X pi@your-pi-address

### Permission denied on shell scripts

chmod +x ~/radio/start_all.sh ~/radio/kill_radio.sh

## Streaming Issues

### Stream starts but listeners hear nothing

- Make sure audio is actually playing on the Pi before starting the stream

- In pi_stream, select the source labelled ▶ ACTIVE — that source has live audio

- Try Auto-detect if unsure which source to use

### darkice exits immediately after starting

- Check the darkice log:  cat ~/.pi_stream_darkice.log

- Most common cause: PulseAudio is not running in the current session

- Make sure you are logged into the desktop, then try again

pulseaudio --check && echo 'running' || pulseaudio --start

### Cannot connect to Icecast2

- Check the service is running:  sudo systemctl status icecast2

- If stopped, start it:  sudo systemctl start icecast2

- Use the Check Icecast button in the pi_stream window to test connectivity

- Make sure port 8000 is not blocked by a firewall

### Wrong password error in darkice log

- The password in pi_stream must match the source-password in /etc/icecast2/icecast.xml

- Default is hackme — if you changed it in Icecast, update it in the pi_stream UI too

### Listeners on other devices cannot connect

- Check Icecast2 is bound to all interfaces — bind-address should be 0.0.0.0 in icecast.xml

- Check your firewall allows port 8000:

sudo ufw allow 8000/tcp  # if ufw is active

- Find your Pi's IP address with:  hostname -I

# Quick Reference

## Starting and Stopping

| Command | What it does |
| --- | --- |
| python3 ~/radio/run_radio.py | Start backend + open desktop UI |
| bash ~/radio/start_all.sh | Same — shell script version |
| bash ~/radio/start_backend.sh | Start backend only (no UI) |
| python3 ~/radio/pi_stream.py | Open the streaming controller |
| bash ~/radio/kill_radio.sh | Stop all radio processes cleanly |
| sudo systemctl start icecast2 | Start the Icecast2 streaming server |
| sudo systemctl stop icecast2 | Stop the Icecast2 streaming server |

## Logs and Status

| Command | What it shows |
| --- | --- |
| cat ~/radio/backend.out | Full radio backend log |
| tail -f ~/radio/backend.out | Radio backend log — live |
| cat ~/.pi_stream_darkice.log | darkice streaming log |
| sudo journalctl -u icecast2 -n 50 | Icecast2 service log |
| sudo systemctl status pi-radio | Radio boot service status |
| sudo systemctl status icecast2 | Icecast2 service status |

## Settings and Config Files

| File | Contents |
| --- | --- |
| ~/.pi_radio_split_settings.json | All radio player settings |
| ~/.pi_radio_library_index.json | Main music library index |
| ~/.pi_radio_show_index.json | Show folders index |
| ~/.pi_radio_commercial_index.json | Commercials index |
| ~/.pi_radio_backend_port | Port the backend is listening on |
| ~/.pi_stream_settings.json | pi_stream Icecast connection settings |
| ~/.pi_stream_darkice.cfg | Auto-generated darkice config (do not edit manually) |
| ~/.pi_stream_darkice.log | darkice streaming log |
| /etc/icecast2/icecast.xml | Icecast2 server configuration |

# Supported Audio Formats

| Extension | Format |
| --- | --- |
| .mp3 | MPEG Audio Layer III |
| .wav | Waveform Audio |
| .ogg | Ogg Vorbis |
| .flac | Free Lossless Audio Codec |
| .m4a | MPEG-4 Audio |
| .aac | Advanced Audio Coding |

Any format VLC can play will work. The list above is what the library scanner recognises by file extension.